When to use a filter cleaner:

- Sand and Zeobrite should be cleaned once per season

- After each algae bloom, sand and Zeobrite should be cleaned

Instructions

1. Turn system off.

2. Pour Rising Sun Pools Filter Cleaner and Degreaser / Regal Filter Cleaner & Degreaser,  into the skimmer.

3. Turn valve to backwash.

4. Turn system on for 1 – 4 seconds, and then turn it off.  Leave the system on just long enough to move product into the filter.  As soon as the liquid leaves the strainer pot, turn the pump off.

5.  Move the handle to the “closed” position (with pump off).  Let the system stand off for, 12 hours. (Regal Filter Cleaner 12 hrs

6.  Turn valve to backwash and turn on the system.  Backwash and rinse until water is clear.

7. Turn valve back to filter (normal mode) and start the system again.

- Always turn pump off when moving handle to different positions.  Always turn handle clockwise