When to use a filter cleaner:

- Sand and Zeobrite should be cleaned once per season

- After each algae bloom, sand and Zeobrite should be cleaned

Instructions

1. Turn system off

2. Remove the strainer lid and pour Sparkling Filter Cleaner Degreaser into the strainer pot. Replace the lid

3. Turn valve to backwash

4. Turn system on for 2 – 5 seconds, and then turn it off.  (Leave the system on just long enough to move product into the filter.  As soon as the liquid leaves the strainer pot, turn the pump off.)

5. With the pump off turn the valve to closed

6.  Let the system stand off for 12 hours.

7.  Turn valve to backwash and turn on the system.  Backwash and rinse as normal.

8. Turn valve back to filter (normal mode) and start the system again

- Always turn pump off when moving handle to different positions.  Always turn handle clockwise