Start Up

1.	Test and adjust alkalinity (80 – 100 ppm) first and then adjust pH (7.2 – 7.6) levels

2.	Shock pool at night with              lbs Super Shock-It or Regal Calcium Hypochlorite (wait 12 hrs before swimming).

- If using regular chlorine granules, mix chlorine into a 5 gallon bucket of water (add chlorine to water not water to chlorine) or add straight to skimmer.

- If using liquid chlorine, use ½ gallon per 10,000 gallons and pour over the pool surface.

3.	Wait 8 hours then add ______ oz of Ionizer Stuff.  Pour over pool surface (15 oz per 10,000 gallons)

4.	Set ion box to the on position so that all lights are on

5.	Let the pool filter for 3 days and bring in water sample to Rising Sun Pools for ensure proper water balance

- Copper levels should be maintained between 0.25 - 0.30 ppm.  When this level is reached, change the ion setting to the auto setting.  Decrease the output to 40% (2 lights).  Let the unit run for a week, then retest the copper levels and make final adjustments to maintain this level.

Weekly

1.	Test and adjust alkalinity (80 – 100 ppm) first, then adjust pH (7.2 – 7.6) and finally test copper levels.

2.	Shock pool every 10 -14 days at night with              Lbs Super Shock-It or Regal Calcium Hypochlorite (wait 12 hrs before swimming).

- If using regular chlorine granules, mix chlorine into a 5 gallon bucket of water (add chlorine to water not water to chlorine) or add straight to skimmer.

- If using liquid chlorine, use ½ gallon per 10,000 gallons and pour over the pool surface.

Helpful Hints

1.	Always shock the pool after heavy bather load or downpour.

2.	Excessive copper and pH levels can cause staining; be sure to check and adjust once a week.

3.   Run filter system a minimum of 12 hours during the daylight hours.

4.   Clean filter sand once a season using Strip Kwik/ Regal Filter Cleaner & Degreaser and after each algae treatment

- Replace sand or Zeobrite in filter every 5-6 seasons in chlorine pools

5.  Have your water tested once a month by Rising Sun Pools to ensure proper balance of water and safe swimming.

6.  Never mix chemicals!! Always use as directed by manufacturer.  Example: Granular chlorine mixed with chlorine tablets will explode!!

7.  Read and follow all directions.  Ask any questions if you are unsure.