SAND CHANGE

SILICA SAND OR ZEOBRITE ONLY

- Turn system off.

- Close all valves or plug the return and skimmer.  (If system is below water level.)

- Remove small drain plug to drain water out of filter tank.

- a. Top mount valve:

Cut PVC or disconnect hoses and remove valve.

b. Side mounted valve:

Remove top dome or remove bolts from center of tank.

- Cover center standpipe with duct tape.

- Remove sand (use a wet and dry vac or scoop it out) be careful not to break the laterals at the bottom of filter

- Flush tank with water.

- Inspect laterals at bottom of tank for cracks or holes.

- Re-install drain plug.

- Make sure standpipe is completely covered.

- Fill tank ½ way with water.

- Add sand. (Tank should be approx. 2/3 full.)

- Replace valve or top of tank. (Inspect o-ring; lubricate or replace.)

- Replace hoses or repair PVC and remove any plugs installed previously.

- Prime pump on BACKWASH and start pump.

**If using silica sand backwash for ~3-5 minutes, rinse for 45 seconds then put back on filter.

**If using ZEOBRITE backwash for ~4 minutes or until sight glass is clear, rinse THEN leave system off for 5 minutes.  Repeat backwash procedure 2 more times until water is running clear from beginning of backwash.

- Check manufacturer instructions for exact amount of sand to be used.  If using Zeobrite use ½ the amount.

- Change sand / Zeobrite every 3-4 seasons in chlorine pools and 2 seasons in Soft Swim pools.