STAIN TREATMENT INSTRUCTIONS

Rising Sun Pools

Make sure chlorine levels are 1.0 ppm or lower before starting treatment.

If NO STAINS are present follow:

Rising Sun Metal Eliminator Treatment

(Do not use if customer has been fighting phosphates)

- Backwash filter

- Allow filter to run continuously throughout treatment

- Pour______ quarts of Metal Eliminator over the pool surface (approx. 16oz. per 10K gallons)

Wait 12 hours

*Can double if high metal content

*Do NOT add chlorine until after Filter Aid is done

- Follow directions for Filter Aid below

_______________________________________________________________________________________________

If STAINS are present follow:

Stain Free / Metal Free

(Use this treatment if customer has been fighting phosphates)

- Backwash filter

- Allow filter to run continuously throughout treatment

- Pour ______ lbs. of Stain Free (1 lbs. per 10K gallons) over surface of pool. Wait 1 hour

- Add ______ Liters of Metal Free over pool surface (1 L per 20K gallons). Wait 24 hours

*Can double if high metal content

*Do NOT add chlorine until after Filter Aid is done

- Brush pool and begin Filter Aid treatment

_______________________________________________________________________________________________

Filter Aid - Simplicity

- Backwash filter

- Set filter on FILTER position

- Slowly pour mixture through skimmer. Allow filter to run continuously throughout treatment

- When filter pressure increased 6 – 8 psi over normal pressure, BACKWASH immediately (this typically takes 2 – 6 hours) DO NOT LEAVE FILTER UNATTENDED!

- Repeat steps 3 and 5 until filter pressure is no longer increasing

- Bring in a water sample to retest minerals

SAND FILTER USAGE CHART

Diameter of filter		12”		18”		24”		30”

Level Cups			1 cup		2.5 cups		4 cups		6 cups

PAY CLOSE ATTENTION TO FILTER PRESSURE!!!! Normal pressure typically between 10 – 20 psi

Filter pressure will increase. When pressure increases 6 – 8 psi over normal BACKWASH FILTER

*May require multiple treatments to eliminate all metals from water

**If the pressure does not go up after first dose of filter aid do not backwash, just add more filter aid.

3-25-09